CN ASSIGNMENT 8

Q1. Configuration and Analysis of BGP Routing

Commands for Router 1:

enable

configure terminal

interface gig0/0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface gig0/0/1

ip address 10.0.0.1 255.255.255.252

clock rate 64000

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

router bgp 100

neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

Commands for Router 2:

enable

configure terminal

interface gig0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

interface gig0/0/1

ip address 10.0.0.5 255.255.255.252

clock rate 64000

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

ip route 192.168.2.0 255.255.255.0 10.0.0.6

end

router bgp 200

neighbor 10.0.0.1 remote-as 100

neighbor 10.0.0.6 remote-as 300

network 10.0.0.0 mask 255.255.255.252

Commands for Router 3:

enable

configure terminal

interface gig0/0/1

ip address 10.0.0.6 255.255.255.252

no shutdown

interface gig0/0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.5

end

router bgp 300

neighbor 10.0.0.5 remote-as 200

network 192.168.3.0 mask 255.255.255.0